| DIGITAL STROBOSCOPE Model : DS-3200 (60.0 to 30,000 RPM/FPM) Model : DS-2200 (60.0 to 18,000 RPM/FPM) Model : DS-1000 (60 to 10,000 RPM/FPM) (Without Display, Rotary adjustment) (AC100V, AC110V, AC220V, AC240V) |  |
| --- | --- |
|  | Keyboard Operating, Wide Range, High Accuracy. A complete digital design for speed measuring, general industrial maintenance, quality control, production, laboratories and for schools and colleges for demonstrating strobe action. Three complete setup memory. Special function "×2" and "÷2" for check the stop motion is actual speed. |

|  | Actual Speed RPM; Stroboscope RPM/FPM; Multiple; Stop Motion Images. 3,000; 12,000; 4; 4. 3,000; 9,000; 3; 3. 3,000; 6,000; 2; 2. 3,000; 3,000; 1; 1. 3,000; 1,500; 1/2; 1. 3,000; 1,000; 1/3; 1. |
| --- | --- |

| DS-3200 | DS-1000 | Rotary RPM/FPM adjustment. Fine adjustment. External trigger. |
| --- | --- | --- |

SPECIFICATION

| Model | DS-3200 | DS-2200 | DS-1000 |
| --- | --- | --- | --- |
| Setup Range | 60.0 - 30,000 rpm | 60.0 - 18,000 rpm | 60.0 - 10,000 rpm |
| Resolution | 0.1 rpm/fpm below 2,000 rpm 1 rpm/fpm above 2,000 rpm |  |  |
| Display | 5 digits LED, 10mm High |  |  |
| Setup Accuracy | ±0.01% |  |  |
| Temp. Drift | ±0.01% |  |  |
| Lamp | Xenon flash lamp |  |  |
| Special Function | Multiplier (×2) flashes, Divider (÷2) flashes Three complete setup memory (M1, M2, M3) Four speeds (Fast, Middle, Slow, Fine) adjustment for quick setting stability. |  | Rotary RPM adjustment. Fine adjustment. External trigger. |
| Power Supply | AC 110V or 220V±10%, 50/60 Hz (Please specify voltage when placing orders.) |  |  |
| Power Consumption | Approximately 25 watts. (Full range) |  |  |
| Operating Temp./Humidity | 0℃ to 50℃ (32℉ to 122℉), Below 85% RH. |  |  |
| Dimension | 115 x 115 x 200 mm. without handle (removable) |  |  |
| Weight | Approximately 1.5Kgs |  |  |
| Standard Accessories | AC power cord, Xenon Lamp, Operation Manual |  |  |
| Optional Accessories | Carrying case (DS-CASE) Xenon Lamp DS-FT-01 (DS-FT-03 for DS-1000) |  |  |

* Appearance and specifications listed in this brochure are subject to change without notice.